|  | Design Procedure Form: Extended Dry Detention Basin |  |  |
| --- | --- | --- | --- |

|  | EXTENDED DRY DETENTION BASIN FEASIBILITY 1. Is the use of an extended dry detention basin appropriate? 2. Confirm local design criteria and applicability. PRELIMINARY HYDROLOGIC CALCULATIONS 3. Compute, WQv volume requirements Compute Runoff Coefficient, Rv Compute WQv Volume requirements 4. Compute site hydrologic input parameters Development Conditions Area CN Adjusted CN Time of concentration 5. Compute WQp peak flow 6. Compute CPv Compute S Compute 1-yr, 24-hr total rainfall depth Compute Qd Compute CPv 7. Compute preliminary release rates Compute WQv release rate Compute CPv release rate EXTENDED DRY DETENTION BASIN DESIGN 8. Pretreatment volume Volpre = Acres of Impervious Area(0.2”)(1’/12”) 9. Water Quality Extended Volume Size extended detention – pool volume is greater of WQv or CPv 10. Conduction grading and determine storage available for extended detention | NOTES: Rv = ______ WQv = ______ acre-ft Pre-developed Post-developed ______ acres ______ acres ______ ______ ______ ______ ______ hours ______ hours WQp = ______ cfs S = ______ Rainfall Depth = ______ inches Qd = ______ inches CPv =__ acre-ft Release Rate = ______ cfs Release Rate = ______ cfs WQpre = ______ acre-ft WQv= ______ acre-ft CPv= ______ acre-ft ED Pool Volume = ______ acre-ft Prepare an elevation-storage table and curve using the average area method for computing volumes. |  |  |
| --- | --- | --- | --- | --- |
|  | Elevation MSL; Area ft2 acres; Average Area Acres; Depth Ft; Incremental Volume Acre-ft; Cumulative Volume acre-ft. |  |
|  | 11. Determine preliminary outlet geometry and stage-discharge relations. 12. WQv Orifice Computations Average ED release rate Average head, h = (ED elev.) / 2 Area of orifice from orifice equation: Q = CA(2gh)0.5 13. Compute release rate for CPv control and Establish CPv elevation Release rate Average head h = (CPvelev.) / 2 Area or orifice from orifice equation: Q = CA(2gh) 0.5 14. Calculate Qp release rate and water surface elevation |  | Release Rate = ______cfs h = ______ft A = ______ ft2 Diameter = ______in WSEL = ______ ft Release Rate = ______cfs h = ______ft A = ______ ft2 Diameter = ______in |  |
|  | Peak stage for the 1-inch, 6-hour storm Peak stage for the 1-yr, 24-hour storm Peak Q10 – Undeveloped Peak Q10 – Developed Peak Q25 – Undeveloped Peak Q25 – Developed Size emergency spillway, calculate 50-year WSEL and set top of embankment elevation 15. Investigate potential extended dry detention basin hazard classification 16. Assess maintenance access and safety features. 17. Attach landscaping plan |  | Peak Stage = ______ ft Peak State = ______ ft Q10-undev = ______ cfs Q10-dev = ______ cfs Q25-undev = ______ cfs Q25-dev = ______ cfs WSEL50 = ______ft Embankment Elevation = ______ft Notes: |  |