|  | Design Procedure Form: Wet pond |  |  |
| --- | --- | --- | --- |

|  | WET POND FEASIBILITY 1. Is the use of a wet pond appropriate? 2. Confirm other design criteria and applicability. PRELIMINARY HYDROLOGIC CALCULATIONS 3. Compute, WQv water quality volume requirements Compute Runoff Coefficient, Rv Compute WQv Volume requirements 4. Compute WQp peak flow Compute modified SCS curve number 5. Compute CPv Compute S (maximum retention) Compute 1-yr, 24-hr total rainfall depth Compute Qd (runoff volume) Compute CPv (chnnnel protection volume) Estimate tc (time of concentration) Estimate qu Compute approximate storage volume 6. Compute release rates Compute WQv release rate Compute CPv release rate 7. Compute site hydrologic input parameters Development Conditions Area CN (SCS curve number) Adjusted CN (curve number adjusted for 1-inch storm) Time of concentration STORM WATER DETENTION BASIN DESIGN 8. Pretreatment volume Volpre = Acres of Impervious Area(0.2”)(1’/12”) 9. Compute Permanent Pool Volume and Water Quality Extended Volume Compute WQv/PPv Compute PPv Size extended detention – pool volume is greater of WQv or CPv 10. Conduction grading and determine storage available for permanent pool (and WQv-ED volume if applicable) | NOTES: Rv = ______ WQv = ______ acre-ft WQp = ______ cfs CN = ______ S = ______ Rainfall Depth = ______ inches Qd = ______ inches CPv =__ acre-ft tc = ______ hours qu = ______ cfs/mi2/inch Storage volume = ______ acre-ft Release Rate = ______ cfs Release Rate = ______ cfs Pre-developed Post-developed ______ acres ______ acres ______ ______ ______ ______ ______ hours ______ hours WQpre = ______ acre-ft WQv/PPv = ______ PPv = ______ acre-ft ED Pool Volume = ______ acre-ft Prepare an elevation-storage table and curve using the average area method for computing volumes. |  |
| --- | --- | --- | --- |
|  | Elevation MSL; Area (acres); Average Area (acres); Depth (ft); Incremental Volume (acre-ft); Cumulative Volume (acre-ft); Volume above Permanent Pool (acre-ft). |  |

|  | 12. WQv Orifice Computations Average ED release rate Average head, h = (ED elev. – Permanent pool elev.) / 2 Area of orifice from orifice equation: Q = CA(2gh)0.5 13. Compute release rate for CPv control and Establish CPv elevation Release rate Average head h = CPv elev. – Permanent pool elev.) / 2 Area or orifice from orifice equation: Q = CA(2gh) 0.5 14. Calculate Qp release rate and water surface elevation | Release Rate = ______cfs h = ______ft A = ______ ft2 Diameter = ______in WSEL = ______ ft Release Rate = ______cfs h = ______ft A = ______ ft2 Diameter = ______in |  |
| --- | --- | --- | --- |
|  | Set up a stage-storage-discharge relationships. |  |  |
|  | Peak stage for (WQv), the 1-inch, 6-hour storm Peak stage for (CPv), the 1-yr, 24-hour storm Peak Q10 – Undeveloped Peak Q10 – Developed Peak Q25 – Undeveloped Peak Q25 – Developed Size emergency spillway, calculate 50-year WSEL and set top of embankment elevation 15. Investigate potential wet detention basin hazard classification 16. Assess maintenance access and safety features. 17. Attach landscaping plan | Peak Stage = ______ ft Peak State = ______ ft Q10-undev = ______ cfs Q10-dev = ______ cfs Q25-undev = ______ cfs Q25-dev = ______ cfs WSEL50 = ______ft Embankment Elevation = ______ft Notes: |  |