|  | Design Procedure Form: Storm Water Wetlands |  |  |
| --- | --- | --- | --- |

|  | STORM WATER WETLAND FEASIBILITY 1. Is the use of a storm water wetland appropriate? 2. Confirm local design criteria and applicability. PRELIMINARY HYDROLOGIC CALCULATIONS 3. Compute, WQv water quality volume requirements Compute Runoff Coefficient, Rv Compute WQv volume requirements 4. Compute WQp peak flow Compute modified SCS curve number 5. Compute CPv Compute S (maximum retention) Compute 1-yr, 24-hr total rainfall depth Compute qd (runoff volume) Compute CPv (channel protection volume) Estimate tc (time of concentration) Estimate qu Compute approximate CPv 6. Size flow diversion structure 7. Compute release rates Compute WQv release rate Compute CPv release rate 8. Compute site hydrologic input parameters Development Conditions Area CN (SCS curve number) Adjusted CN (curve number adjusted for 1-inch storm) Time of concentration STORM WATER WETLAND DESIGN 9. Pretreatment volume Volpre = Acres of Impervious Area(0.2”)(1’/12”) 10. Allocate permanent pool and temporary pool storage volumes based on design requirements. 11. Allocate the open water, high marsh, and low marsh zones. Area open = 0.10 (BMParea) Area high marsh = 0.45 (BMP area) Area low marsh = 0.35 (BMParea) 12. Set design elevations and dimensions of facility 13. WQv Orifice Computations Average ED release rate Average head, h = (ED elev.–Permanent pool elev.)/2 Area of orifice from orifice equation: Q = CA(2gh)0.5 Compute release rate for CPv control and Establish CPv elevation Release rate Average head h = CPvelev.–Permanent pool elev.)/2 Area or orifice from orifice equation: Q = CA(2gh) 0.5 Set Up Stage-Storage-Discharge Relationship For Routing 14. Route flows through bioretention facility. Peak stage for the 1-inch, 6-hour storm Peak stage for the 1-yr, 24-hour storm Peak Q10 – Undeveloped Peak Q10 – Developed Peak Q25 – Undeveloped Peak Q25 – Developed 15. Design conveyances to facility (off-line systems) 16. Size emergency spillway, calculate 50-year WSEL and set top of embankment elevation. 17. Investigate Potential pond/wetland hazard classification. 16. Prepare vegetation and landscaping plan. | NOTES: Rv = ______ WQv = ______ acre-ft WQp = ______ cfs CN = ______ S = ______ Rainfall Depth = ______ inches Qd = ______ inches CPv =__ acre-ft tc = ______ hours qu = ______ cfs/mi2/inch CPv = ______ acre-ft Release Rate = ______ cfs Release Rate = ______ cfs Pre-developed Post-developed ______ acres ______ acres ______ ______ ______ ______ ______ hours ______ hours WQpre = ______ acre-ft WQv (storage volume) = at elev. CPv (storage volume) = at elev. Area open = ______ acre Area high marsh = ______ acre Area low marsh = ______ acre Length = ______ ft Width = ______ ft Elevation top of facility = ______ ft Other elevations = ______ ft = ______ ft = ______ ft Release Rate = ______cfs h = ______ft A = ______ ft2 Diameter = ______in WSEL = ______ ft Release Rate = ______cfs h = ______ft A = ______ ft2 Diameter = ______in Peak Stage = ______ ft Peak State = ______ ft Q10-undev = ______ cfs Q10-dev = ______ cfs Q25-undev = ______ cfs Q25-dev = ______ cfs = ______ Online or ______ Offline? WSEL50 = ______ft Embankment Elevation = ______ft Length = ______ ft Length of Weir (if used) = ______ ft Notes: |  |
| --- | --- | --- | --- |