|  | Design Procedure Form: Sand Filters |  |  |  |
| --- | --- | --- | --- | --- |
|  | SAND FILTER FEASIBILITY 1. Is the use of a sand filter appropriate? 2. Confirm local design criteria and applicability. PRELIMINARY HYDROLOGIC CALCULATIONS 3. Compute, WQv water quality volume requirements Compute Runoff Coefficient, Rv Compute WQv volume requirements 4. Compute site hydrologic input parameters Development Conditions Area CN (SCS curve number) Adjusted CN (SCS curve number adjusted for 1-inch storm event) Time of concentration 5. Compute WQp peak flow 6. Compute CPv (channel protection volume) Compute S (maximum retention) Compute 1-yr, 24-hr total rainfall depth Compute qd (runoff volume for 1-year, 24-hour storm event) Compute CPv 7. Size flow diversion structure 8. Compute release rates Compute WQv release rate Compute CPv release rate SAND FILTER DESIGN 9. Compute sedimentation chamber volume Volpre = (0.20)(WQv) 10. Size filtration bed chamber Compute area from Darcy’s Law 11. Develop stage-storage and stage-discharge relationships 12. Route Flows through sand filter 13. Design inlets and underdrain system, 14. Compute overflow weir sizes Compute overflow – Orifice equation Weir from sedimentation chamber – Weir equation Weir from filtration chamber – Weir equation 15. Assess Maintenance Access and Safety Features |  | NOTES: Rv = ______ WQv = ______ acre-ft Pre-developed Post-developed ______ acres ______ acres ______ ______ ______ ______ ______ hours ______ hours WQp = ______ cfs S = ______ Rainfall Depth = ______ inches qd = ______ inches CPv =__ acre-ft Release Rate = ______ cfs Release Rate = ______ cfs Volpre = ______ acre-ft Ar = ______ft2 L = ______ft W = ______ft Elev; Area. (ft2); Volume. (ft3); Acc. Vol. (ft3); Q (cfs). Q =______cfs Length = ______ft Length = ______ft |  |
|  | Notes: |  |  |  |