#### SP-XX, WOOD PRIVACY FENCE

Version Date:  11/10/2025					Revision Date:  MM/DD/YYYY by XXX

1.0	DESCRIPTION

Work covered by this special provision consists of furnishing and erecting wood privacy fence and gates (where applicable) as shown on the plans and in accordance with the specifications herein.

2.0	MATERIALS

The Contractor will inspect the existing fence and gates (where applicable) to determine the materials and replace to match unless otherwise specified on the drawings. At a minimum, fence materials should meet the following specifications derived from ASTM F537.

(a)  Wood:

(1)   All materials used should be either naturally rot resistant wood (such as Cedar), pressure treated for rot-resistance or be coated thoroughly with paint or a protective coating immediately after installation.

(2)   Planks/Pickets(planks) shall be 5/8" minimum thickness.

(b) Concrete:  Use Class B concrete for anchors. Instead of Class B concrete, pre-mixed commercially bagged dry concrete mix may be used if the concrete meets the minimum strength requirements for Class B concrete when mixed with the quantity of water shown on the instructions printed on the bag.

(c)   Fasteners; Fasteners shall be made of a non-rusting, non-corrosive material, or coated to resist rusting. Fasteners shall be long enough to penetrate the receiving member twice the thickness of the thinner member but not less than one and one-half inches (1-1/2").

3.0	CONSTRUCTION METHODS

Construct fence so that it is plumb, vertically straight and the top of the planks will form a continuous line with no dips or abrupt vertical changes.

- Post spacing shall not exceed eight feet (8') on center.  Post should be set a minimum of thirty inches (24") deep for fences up to six feet in height (6'). All terminal, corner and gate posts should be set thirty-six inches (36") deep.

- Post holes should be at least four inches (4") larger in diameter than the largest dimension of the post. All posts should be set in concrete.

- All materials are to be securely fastened (i.e. vertical boards to horizontal stringers, stringers to vertical posts) and be free from rot, rust, vandalism and other sources or decay.  Wood that is warped or structurally unsuitable shall not be used.

- Stringers: Stringers shall be 2x4 minimum (nominal). Fences up to five feet in height (5') shall have a minimum of two rails (stringers) top and bottom. Fences over five feet in height (5') shall have a third rail at center height.  Vertical posts and the stringers should be installed on the inside of the fence, not facing the street or adjacent properties.

- Planks must be fastened to horizontal bracing stringers running from vertical post to post. Planks must also have at least a one inch (1”) gap between the ground and the wood to prevent rotting and decay.

- Solid wood fences should allow for expansion to avoid buckling as follows: For plank widths 2 to 4 inches, a one-sixteenth inch (1/16") space should be provided; for widths 6 to 8 inches, a minimum space of one-eighth inch (1/8") should be provided.  Expected shrinkage of freshly pressure treated material may be considered to reduce initial installation spacing.

- The tops of the planks/pickets may be cut to many different designs such as dog ear (corners cut off at 45 degree angles), gothic (two arc shaped cuts that meet in the center to form a graceful point), standard point (boards cut to a point with 45 degree angles), or domed or rounded top (where the top is cut to a half circle). There are many shapes and styles that may be custom cut at specific request. Post tops may also be cut to several decorative shapes such as pyramid, gothic, domed and chamfered. Also, shaped terminals may be screwed onto the tops of posts.

4.0	MEASUREMENT

The quantity of wooden fence to be measured will be the actual number of linear feet of fence, measured from center of post to center of post, which has been completed and accepted.  The quantity of gates to be measured will be per each gate which has been completed and accepted.  There will be no separate measurement of posts.

5.0	PAYMENT

The quantity of wooden fence, measured as provided above, will be paid for at the contract unit price per linear foot for "Fence, Wooden".  Such payment will be full compensation for all work of furnishing and installing proposed fence to include but not limited to terminal posts, line posts, stringers, slats or panels, fasteners, and all incidental materials. The quantity of gates to be paid for will be per each gate which has been completed and accepted.  There will be no separate payment of posts.

Payment will be made under:

FENCE, 4’ WOOD PRIVACY FENCE ……………………………………………………………………………………..………………….............LF

FENCE, 4’ WOOD (GATE)...………………………………………………………………………………………………………………..….............EA

FENCE, 5’ WOOD PRIVACY FENCE …………………………………………………………………………………….…………………..............LF

FENCE, 5’ WOOD (GATE)……....………………………………………………………………………….……………………………..….............EA

FENCE, 6’ WOOD PRIVACY FENCE …………………………………………………………………………………….…………………..............LF

FENCE, 6’ WOOD (GATE)….....………………………………………………………………………….………………………………..…............EA